Controller LAN Communication

- Communication Port: TCP- 8883

  - 8883 must be unblocked, it is the MQTT port

  - 443 or 80  is HTTPS for browser, in most cases (to allow user access to the internet)

- IoT Endpoint Name: a3ctijj2jmv8d5-ats.iot.us-east-1.amazonaws.com

- Get MAC address from controller screen, if that is required, to enable that MAC

  - ACC2, view controller Settings menu, Networking screen to get MAC if necessary

  - ICC2, must know IP address to get to internal webpage

    - DHCP must be turned on for LAN to get IP

    - Type IP address to access internal webpage to get the MAC, if necessary

    - Computer accessing the controller for configuration must be within same network for this part of setup

Static IP:

- Centralus is a cloud-based system.

  - It is not an Enterprise system, and does not have or recognize Enterprise security (WPA). It normally works with DHCP to follow dynamic addressing, as necessary.

- If they want to assign a static IP, first step is to get the MAC as bulleted above.

- Then they can assign MAC to static IP if that is the requirement.